# Control Plane Comparison

John R. Naylor, November 2024

# Governance

| Category | Ember+ | AES70 | Catena | NMOS IS-x Suite | Comments |
| --- | --- | --- | --- | --- | --- |
| References | [2], [3] | [1] | [4] [5] | [6] |  |
| Standard | No | Yes, AES | Yes, SMPTE ST-2138-x, in progress | No |  |
| Industry Sponsors | Lawo | AES | IABM, Control Plane WG, SMPTE | AMWA |  |
| Open Source | Yes | Several, but many not active ocac – C99 | Yes | Yes Implementations available by Sony, Nvidia | AES70 has more repos – Paul Treleaven has better info |
| Public SDK | Yes | Yes – need reference | Yes | No |  |
| Languages | C++ | C | C++, Java | JSON-schema |  |
| License | Boost Software License V1.1 | ocac - MIT | BSD 3-Clause | Apache 2 |  |

# Technical

| Category | Ember+ | AES70 | Catena | NMOS IS-12 | Comments |
| --- | --- | --- | --- | --- | --- |
| Schemas for Protocol Objects | Yes, Abstract Syntax Notation | Unknown | Yes, protobuf IDL and JSON-schema | Yes, JSON-schema |  |
| Serialization / object encoding | Yes, EmBER (embedded basic encoding rules), binary format | Yes, binary (OC1?). JSON and XML planned | Yes, binary and JSON | JSON |  |
| Defines Network Message Formatting | Yes, S101 packetization protocol | Unknown | No, deferred to connection manager. Support for gRPC and REST | Unknown |  |
| Transport Agnostic | Yes | Yes | Yes | Unknown |  |
| Device Partitioning | No | Unknown | Yes | Unknown | e.g. present a 4M/E switcher as 2 2M/E switchers |
| Reference sub-devices | Unknown | Yes | Yes, device models can import definitions via URL | Yes |  |
| Plug & Play | Yes | Yes | Yes | Unknown | i.e. clients can be “dumb” but provide reasonable GUIs without prior knowledge of devices they meet online |
| Performance & Scaling |  |  | Subscriptions to minimize traffic |  |  |

# Dependencies

| Category | Ember+ | AES70 | Catena | NMOS IS-12 | Comments |
| --- | --- | --- | --- | --- | --- |

# Security

| Category | Ember+ | AES70 | Catena | NMOS IS-12 | Comments |
| --- | --- | --- | --- | --- | --- |
| Secure Comms (TLS) | No | Yes, relies on transport to provide this | Yes, relies on HTTP/1.1 or HTTP/2 to provide this | Yes, BCP-003 | Confidentiality Integrity |
| Zero Trust | No | No | Yes, defines access scopes for monitor, operate, configure, and administer workflows. OAuth2 based | Yes, via IS-10 profile for OAuth2 | Non-repudiation Fine-grained Access Control |
| Transport Redundancy | Unknown | Yes (is this -7? Ask Jeff) | No, delegated to cloud architectures such as load balancers, service mesh, proxies | Unknown | Availability |

# Internationalization

| Category | Ember+ | AES70 | Catena | NMOS IS-12 | Comments |
| --- | --- | --- | --- | --- | --- |
| Multi-language Support | No | No | Yes | Unknown |  |

# Market Adoption & Maturity

| Category | Ember+ | AES70 | Catena | NMOS IS-12 | Comments |
| --- | --- | --- | --- | --- | --- |
| Publication Date | 2012 | 2015 | 2025 (planned) | V1.0 2024 |  |
| Users | 30 companies listed on open source site | Unknown | As an evolution of openGear protocol, there are 150 openGear Partners who could readily transition to Catena | Unknown |  |

# Benchmarks

| Category | Ember+ | AES70 | Catena | NMOS IS-12 | Comments |
| --- | --- | --- | --- | --- | --- |
| Embedded Host w/o Crypto Acceleration |  |  |  |  | We need to agree on a useful metric – messages / second, CPU, Memory, Network resource usage |
| Embedded Host with Crypto Acceleration |  |  |  |  |  |
| Server Class Host |  |  |  |  |  |
| Ground - Cloud |  |  |  |  |  |

# Bibliography

| [1] | J. Berryman, AES70, Open Control Architecture, Quick View, 2024. |
| --- | --- |
| [2] | Lawo, "Ember+ Specification," [Online]. Available: https://github.com/Lawo/ember-plus/blob/master/documentation/Ember%2B%20Documentation.pdf. [Accessed 1 July 2024]. |
| [3] | Skyline Communications, "Dataminer Device Specific Integrations," [Online]. Available: https://docs.dataminer.services/develop/DSI/DSIEmberPlus/DSIEmberPlus.html. [Accessed 13 November 2024]. |
| [4] | SMPTE, "34CS-WD- ST-2138-10 Catena Model.docx," 6 November 2024. [Online]. Available: https://smpte.sharepoint.com/:w:/s/RIS-OSA-Catena/ERQy6HRqZeBJm6U2AwTX4YIBZ-OBx-3vwO3pTNO-ZPd8jQ?e=h6mP3p. [Accessed 13 November 2024]. |
| [5] | SMPTE, "34CS-WD- ST-2138-11 Catena gRPC Connection Type.docx," 11 November 2024. [Online]. Available: https://smpte.sharepoint.com/:w:/s/RIS-OSA-Catena/EQJNiPovgcpCjN2v1jwNYw8BvkKoRXzJV4zkJlroE45nuQ?e=qdAogJ. [Accessed 13 November 2024]. |
| [6] | AMWA, "AMWA IS-12 NMOS Control Protocol," [Online]. Available: https://specs.amwa.tv/is-12/. [Accessed 13 November 2024]. |