# Control Plane Comparison

John R. Naylor, November 2024

AES70 column updated by AES writing group SC-02-12-L on 2024.12.06

# Governance

| Category | Ember+ | AES70 | Catena | NMOS IS-x Suite | Comments |
| --- | --- | --- | --- | --- | --- |
| References | [2], [3] | [1] | [4] [5] | [6] |  |
| Standard? | No | Yes, from AES: AES70-1-2024 AES70-2-2024 AES70-3-2024 AES70-4-2024 (WIP) AES70-21-2024 (WIP) AES70-22-2024 (WIP) | Yes, SMPTE ST-2138-x, in progress | No |  |
| Industry Sponsors | Lawo | AES + member companies of OCA Alliance | IABM, Control Plane WG, SMPTE | AMWA |  |
| Open Source | Yes | Variety of resources - portal page here. Three new projects are currently active. | Yes | Yes Implementations available by Sony, Nvidia |  |
| Public SDK | Yes | No formal SDK. Older reference implementation here, new reference implementation WIP in OCA Alliance. Various commercial libraries here: http://deuso.de/aes70/ | Yes | No |  |
| Languages | C++ | C++,C#,jscript, python (WIP), swift (WIP) | C++, Java | JSON-schema |  |
| License | Boost Software License V1.1 | Free software is Apache 2 or MIT. No copyleft licenses. | BSD 3-Clause | Apache 2 |  |

# Technical

| Category | Ember+ | AES70 | Catena | NMOS IS-x Suite | Comments |
| --- | --- | --- | --- | --- | --- |
| Schemas for Protocol Objects | Yes, Abstract Syntax Notation | UML (XMI format available) | Yes, protobuf IDL and JSON-schema | Yes, JSON-schema |  |
| Serialization / object encoding | Yes, EmBER (embedded basic encoding rules), binary format | Binary ("OCP.1"); JSON/XML ("OCP.2") is in late-stage draft. | Yes, binary and JSON | JSON |  |
| Defines Network Message Formatting | Yes, S101 packetization protocol | Yes, for both binary and JSON/XML. | No, deferred to connection manager. Support for gRPC and REST | Unknown |  |
| Preferred Transport | S101 | TCP, SSL, UDP, WebSocket, or point-to-point link. | HTTPS/2 (gRPC) HTTPS/1.1 (REST) | HTTPS |  |
| Device Partitioning | No | No | Yes | Ask the experts | e.g. present a 4M/E switcher as 2 2M/E switchers |
| Reference sub-devices | Unknown | Yes, via the "reusable block" feature. | Yes, device models can import definitions via URL | Yes: IS-12 | e.g. there’s a reference 4-band eq definition that can be imported by devices wishing to have 4-band eq controls |
| Self-describing? | Yes | Depends on controller. AES70 supports full discovery and enumeration, so any degree of controller automation is possible. | Yes | IS-12 is self-describing | i.e. clients can be “dumb” but provide reasonable GUIs without prior knowledge of devices they meet online |
| Discovery | EmBER Service Discovery_v0.2.docx | Normally mDNS/DNS-SD | In design – also looking at DNS-SD | Based on DNS-SD via IS-04 |  |
| Performance & Scaling |  | High network efficiency. Event-driven: no polling, ever. Nominal design goal is at least 10,000 devices. | Subscriptions to minimize traffic |  |  |
| Web Client Direct Access | No (S101) | Yes. For example, see the open-source library aes70.js, here. | gRPC: no REST: yes | Yes. IS-12 (websocket), IS-07 |  |
| Deterministic | Unknown | No, but protocol procedures are not processing-intensive, which allows fast response. Also, timed task scheduling features are available. | Unknown | IS-07 is time-stamped | If an operator presses a button the associated action happens within a defined and repeatable time. |
| Port Agility | Unknown | No particular port constraints; standard conforms to normal IP practice. | Yes (gRPC) | Unknown | Ability for devices to listen on configurable IP ports |
| Ease of integration | Unknown | AES70 is generally sensible and regular. One experienced implementer got it working in one day, using available tools and libraries. | Intel got a service integrated same-day | Becoming increasingly easier as the specs and implementations mature with field-exposure. | How long to go from zero to working device |

# Dependencies

| Category | Ember+ | AES70 | Catena | NMOS IS-x Suite | Comments |
| --- | --- | --- | --- | --- | --- |
| Hard Cost Items |  | None |  |  | e.g. patent licenses |
| RAND |  | None |  |  | Reasonable and non-discriminatory |

# Security

| Category | Ember+ | AES70 | Catena | NMOS IS-x Suite | Comments |
| --- | --- | --- | --- | --- | --- |
| Secure Comms (TLS) | No | Yes, relies on SSL | Yes, relies on HTTP/1.1 or HTTP/2 to provide this | Yes, BCP-003 | Confidentiality Integrity |
| Zero Trust | No | No | Yes, defines access scopes for monitor, operate, configure, and administer workflows. OAuth2 based | Yes, via IS-10 profile for OAuth2 | Non-repudiation Fine-grained Access Control Support for Roles |
| Transport Redundancy | Unknown | Yes. Flexible network model allows any degree of redundancy. | No, delegated to cloud architectures such as load balancers, service mesh, proxies | Unknown | Availability |
| NAT Traversal |  | No special features. Using the WebSocket transport option will allow browser-style net traversal. |  |  | Device can reach out to establish bi-directional comms. One implementation could be an MQTT broker at a public endpoint |
| Registration Access Control |  | No features. |  | Yes – via IS-10 access to IS-04 end-points. But is it used? |  |
| IAM Integration Support for Service accounts |  | No features. |  |  | Identity and Access Management |
| Fail open/closed |  | No features. |  |  |  |

# Internationalization

| Category | Ember+ | AES70 | Catena | NMOS IS-x Suite | Comments |
| --- | --- | --- | --- | --- | --- |
| Devices / Services provide Multi-language Support for UIs | No | No (check again with JB) UTF-8 character set is used. | Yes | Unknown |  |
| Specifications / Standards available in multiple languages |  | Protocol specification is currently in English and Chinese. |  |  |  |

# Market Adoption & Maturity

| Category | Ember+ | AES70 | Catena | NMOS IS-x Suite | Comments |
| --- | --- | --- | --- | --- | --- |
| Publication Date | 2012 | 2024 | 2025 (planned) | V1.0 2024 |  |
| Users | 30 companies listed on open source site | List of known AES70 products is here. About 80 products from 12 manufacturers. At least four more manufacturers are working on AES70 products. Current focus is on professional audio. | As an evolution of openGear protocol, there are 150 openGear Partners who could readily transition to Catena | ST-2110 infrastructure. AR has list of nmos supported devices – will provide link here. |  |

# Benchmarks

| Category | Ember+ | AES70 | Catena | NMOS IS-x Suite | Comments |
| --- | --- | --- | --- | --- | --- |
| Embedded Host w/o Crypto Acceleration |  | Anecdotal information only, e.g. Bosch uses a 500-node test system to verify <10 ms response times. |  |  | We need to agree on a useful metric – messages / second, CPU, Memory, Network resource usage |
| Embedded Host with Crypto Acceleration |  | No information |  |  |  |
| Server Class Host |  | See note above. |  |  |  |
| Ground - Cloud |  | No information. |  |  |  |

# Bibliography

| [1] | J. Berryman, AES70, Open Control Architecture, Quick View, 2024. |
| --- | --- |
| [2] | Lawo, "Ember+ Specification," [Online]. Available: https://github.com/Lawo/ember-plus/blob/master/documentation/Ember%2B%20Documentation.pdf. [Accessed 1 July 2024]. |
| [3] | Skyline Communications, "Dataminer Device Specific Integrations," [Online]. Available: https://docs.dataminer.services/develop/DSI/DSIEmberPlus/DSIEmberPlus.html. [Accessed 13 November 2024]. |
| [4] | SMPTE, "34CS-WD- ST-2138-10 Catena Model.docx," 6 November 2024. [Online]. Available: https://smpte.sharepoint.com/:w:/s/RIS-OSA-Catena/ERQy6HRqZeBJm6U2AwTX4YIBZ-OBx-3vwO3pTNO-ZPd8jQ?e=h6mP3p. [Accessed 13 November 2024]. |
| [5] | SMPTE, "34CS-WD- ST-2138-11 Catena gRPC Connection Type.docx," 11 November 2024. [Online]. Available: https://smpte.sharepoint.com/:w:/s/RIS-OSA-Catena/EQJNiPovgcpCjN2v1jwNYw8BvkKoRXzJV4zkJlroE45nuQ?e=qdAogJ. [Accessed 13 November 2024]. |
| [6] | AMWA, "AMWA IS-12 NMOS Control Protocol," [Online]. Available: https://specs.amwa.tv/is-12/. [Accessed 13 November 2024]. |
| [7] | AES, "AES70-1-2024: AES standard for Audio applications of networks - Open control architecture - Part 1: Framework", available through AES Standards Store. https://www.aes.org/publications/standards/ |
| [8] | AES, "AES70-2-2024: AES standard for audio applications of networks - Open Control Architecture - Part 2: Class structure", available through AES Standards Store. https://www.aes.org/publications/standards/ |
| [9] | AES, "AES70-3-2024: AES standard for audio applications of networks - Open Control Architecture - Part 3: OCP.1 Binary protocol", available through AES Standards Store. https://www.aes.org/publications/standards/ |